Appendix 28: CCTV and other sensor requirement

- CCTV

  - CCTV hardware requirement (requirements from AEO)：

- Assurance of 24 hours/365 days a year site protection and prevent fraud (security officials, cameras, Yard fence to ensure dropped trailers are secure).

- CCTV monitoring should cover the entire warehouse and also the entrance of the park.

- The CCTV should have the capacity to remain the monitoring record for at least three months

- CCTV pixels should no less then 4 million

  - CCTV集成要求 ( requirements from digital twin)：

- 厂内摄像头的CAD图纸或布局图，要求图上要标注每个监控的唯一编号（以监控厂家的安装摄像头后的唯一ID为准）

- 提供摄像头清单，清单内需包含每个摄像头的唯一编号、名称、类型、IP地址和RTSP视频流地址

- 提供监控系统数据对接文档，用来对接设备状态、告警事件和统计数据

- CCTV监控系统需要具备中央管理端，能够将监控视频流向外部系统提供信息源

- CCTV监控中央管理端能够与互联网进行视频流传输且有网络安全防护机制

- Temperature and humidity sensor ( requirements from digital twin)：

- 测量范围：温度-20-60℃；湿度0-99%RH

- 测量精度：温度±0.5℃；湿度±3%RH

- 采样速率：2S/次

- 供电方式：外部供电5VDC，内置电池管理系统

- 静态功耗：休眠小于1mA,正常工作3mA

- 通讯方式：USB、GPRS、GPS、支持MQTT协议，TCP固定协议上传

- Thermal camera ( requirements from digital twin)：

- 支持火点探测报警，支持冷热点报警和工业测温报警功能；

- 支持声光报警联动，支持双目融合；

- 支持IP白名单和黑名单、MAC白名单与黑名单、多级用户管理，人性化监控保密和权限管理；

- 支持3路报警输入、2路报警输出功能，最大支持256GB Micro SD卡

- AGV ( requirements from digital twin)：

If contractor proposed AGV, it should meet below requirement to fulfill digital twin standard:

  - 数字孪生对AGV的需求信息

- AGV行动路线：可以坐标形式提供，即实时位置，更新时间在100ms以内

- 告警状态：例如设备运行中/是否载货/充电中/故障等信息可以提供，反馈方式可以运行码反馈形式

  - 信息接口

- 可由WCS主动推送，建议时间间隔100ms

- AGV可提供API接口，需要AGV WCS的信息投射到公网上